Automated Technology Migration Methodology for Mixed-Signal Circuit Based on Multi-Start Optimization Framework

Liuxi Qian, Zhaori Bi, Dian Zhou and Xuan Zeng

Abstract— Optimization-simulation loop based method is popular and efficient in design migration/reuse automation. However, it is only restricted to be used in block-level due to the complexity of current mixed-signal system. This paper presents a hierarchical methodology for efficiently migrating mixed-signal circuit design from one technology node to another while keeping the same circuit and layout topologies. It utilizes two stages of optimization processes to automatically resize and refine device dimensions in target technology. In the first stage, in order to avoid the costly simulation time without scarifying systematical functionality, only one block is represented in transistor-level, while other blocks are replaced with behavioral models. The multi-start global optimization technique is applied to resize the transistor-level block in systematic connection.  This stage provides a good initial point for next system-level refinement. Moreover, for obtaining a process and parasitic closure solution, both parasitic and process variation effects are explored and used to constrain the schematic migration. A representative mixed-signal system, Charge Pump Phase-Locked Loop, is used to validate the proposed methodology. The experimental results show that the proposed methodology efficiently generates quality designs in target technology with much less simulation iterations, when comparing with recent available approaches.

Index Terms—technology migration, design reuse, Phase-Locked Loop (PLL), optimization method.

# I.		Introduction

N

owadays, the complex mixed-signal design integrates with a mix of digital, analog and Radio Frequency (RF) blocks. As the rapid improvement of process technology, in order to exploit the benefits of the scaled technologies, the existing mixed-signal design is porting from original technology to another while keeping the same schematic and layout topologies. During design migration, experienced designers are involved to manually readjust the device sizes to pull the design into compliance over all the required conditions. However, considering circuit complexity and tight time-to-market constraint, current mission of technology migration is to reuse the proven functional blocks as many as possible, rather than redesign every block from scratch.

Due to the aid of standard cell based design methodology and advanced Electronic design automation (EDA) tools, technology migration for digital circuit can be achieved by rerunning the fully automated synthesis flow [1]. The remap for analog/mixed-signal circuit, on the other hand, is still cumbersome and resource-intensive so far. Because of the highly nonlinear relationship between the device parameters and circuit characteristics, most analog/ mixed-signal circuits are designed for a specific technology, which greatly limits the flexibility to be reused between different process nodes. Simply employing the same shrink rule as migrating digital circuit, the ported analog/mixed-signal circuit cannot even function correctly. Moreover, the performance metrics, in the most of time, are changed accordingly under target technology. Therefore, a complete redesign is often unavoidable and tedious resizing task is inevitable.

In the literature, numerical optimization algorithms are intensively employed in the automation procedure of technology migration and design reuse for topology synthesis, device resize and biasing in order to meet all the design specs [2-7, 15, 17-22]. However, there still exist the following incommodities: 1) most approaches separately treat a complex system block by block without an overall systematic consideration. Note that a well-designed block can be incompatible to the whole system. The key hurdle that prevents the transition from cell-level to system-level is that system-level design is associated with a large number of design variables, resulting in an incontrollable search space; 2) the optimization algorithms utilized by most approaches have either initial points dependency problem, like gradient-based algorithms [6], or slow convergence problem, such as genetic algorithms [20-21], geometric programming [22-23], simulated annealing [19], and so on. 3) most existing process migration approaches are failing to consider circuit reliability and robustness. In fact, given the Process, Voltage, and Temperature (PVT) variations, circuit characteristics can easily deviate from desired values. 4) most of existing methods for process migration focus on schematic level migration and fail to account for the layout parasitics.

Based on above mentioned issues, this paper presents an efficient solution for migrating mixed-signal design from one CMOS technology to another. The major characteristics of the proposed method are summarized as follows:

a. Systematic evaluation. Firstly, a Transistor-Level (TL) block in systematical configuration is retargeted, while other blocks are represented in Behavioral-Level (BL). For one aspect, this abstraction reduces the simulation time. For another, it assures the functional accuracy as a whole system. Then, starting from the result determined in stage 1, another systematic level resizing is conducted.

b. Global optimization. The Multi-start Global Optimization

(MGO) framework is employed for automatically sizing TL blocks in target technology in stage 1. This strategy presents fast convergence ability and overcomes the start-point dependency and local-optimal trapping problems.

c. Robustness verification. The PVT corners are considered inside migration process to achieve a robust design.

d. Parasitic aware evaluation. In order to obtain parasitic-closure design, the impacts of layout parasitics is explored and the extracted parasitics are reused to constrain the optimization based resizing procedure.

The remaining parts of this paper are organized as follows. In Section II, existing approaches for technology migration and the exampled mixed-signal circuit structure are reviewed. A detailed description of proposed automatic technology migration methodology is presented in III. In Section IV, the experimental results to validate the proposed approach are provided. Finally, a conclusion is offered in Section VI.

# II.		Related works and case study

## Related works

There are many automatic and computer-aided technology migration approaches available in the literature, which can be broadly classified into three categories:

1) Analytical equation/linearized equivalent model based approach. In [4], the equivalent linearized circuit model is first abstracted by using operation point analysis method. Then optimization iterations are used to minimize the differences of the equivalent circuit element parameters between source and target technologies.  In [7], transistor’s transconductance gm and output conductance gds are kept as close as possible with respect to the original design in order to preserve small signal frequency behaviors. Both scaling equations and numerical optimization iterations are used to adjust transistor dimensions to match the same gm and gds values between the original and scaled circuits. In [8], by assuming the same supply voltage, initial scaling factors are firstly calculated based on the constant transconductance constraint of minimal transistor length. Then device sizes are tuned based on qualitative reasoning method, where user-provided qualitative dependency matrix is required. The qualitative dependency matrix maintains the relationship between important parameters and circuit features, which are based on the designers’ experiences or simple design equations. Paper [9] assumes that preserving the parameters of each individual component in a circuit would preserve the overall circuit features. Then, the device sizes are tuned by keeping relative bias conditions during retargeting process. In [10] and [11], scaling factors for some parameters, like transistor dimension and transconductance are computed based on advanced compact model [12] of MOSFET transistors. In [13], generalized scaling factor based resizing rules are derived for maintaining the original dynamic range and gain-bandwidth specs. In [14], the scaling rules are proposed based on the simplest MOS transistor model (level 1 model) for keeping the same figure of merit and reducing area and power consumption, if the target technology is smaller.

Fig.1 Simulation-optimization loop based methodology

The analytical equation/linearized equivalent model based approach attempts to achieve the same characteristics as the original design by using simplified design equations or linearized models. It starts from original designs and gathers the necessary information connecting target and source technology. This method has the principal advantage of fast evaluation in terms of computational time. However, both manufacturing processes and circuit designs nowadays have been an exponential growth of complexity.  It often faces challenges in accuracy due to the approximated and simplified equations/models only describe the essential dynamics of the circuit instead of representing the circuit in practice. Considering recent nano-scaled integrated circuit, its complete performance evaluation often requires a combination of dc, ac, transient analysis, etc., which are unrealistic to be captured through analytical equations.  This type of method trades accuracy for speed, making it fail to migrate into mainstream use in industry.

2) Template based approach. In [15], a generator approach is proposed. A set of basic analog building cells, like differential pair, current mirror, is implemented in a scalable and technology independent manner by using generic model approach. The complex analog/mixed-signal design is firstly transferred into a compact symbolic description and then resized through optimization iterations. For a certain type of circuit, like digital/analog converter, a module generator based approach is developed in [16]. Basic building blocks are made firstly into standard cells. Then hierarchical sizing procedures are used to map system-level   parameters   into TL parameters.    The generation of system-level model is based on the design experience, while the TL synthesis is based on analytical equations. In [17], an analog IP database is firstly built, which includes adaptive models, circuit characteristic equations and variational ranges. Then, an ASIC-like design flow for analog reuse is implemented based on the analog IP database.

The template based method covers a substantial fraction of circuit needed in most applications. The structures under migration are considered as a set of basic cells. However, a more general approach which is able to cope with arbitrary architecture is still in great demand.

3) Simulation-optimization loop based approach. This approach, as shown in Fig.1, sets up a consistent platform for the collaborative work between mixed-signal circuit simulator and global optimizer. It adjusts the circuitry parameters under the guidance of an optimization algorithm and evaluates each sized circuitry using the same circuit simulator as it is designed, in order to comply with the desired performance goals. This method requires less or no expert knowledge of the designed circuit for creating mathematical equations or equivalent linearized models. It is based on more reliable and practical

Fig.2 Schematic of CPPLL: (a) PFD; (b) Charge pump; (c) Loop filter; (d) VCO; (e) Block diagram

Fig.3 CPPLL block diagram

Simulation Program for Integrated Circuit Emphasis (SPICE)  accurate simulations. Furthermore, this approach provides great generality which is applicable to different types of circuits, technology nodes and design specs. Last but not least, the associated optimization procedure generates new designs automatically, which will not only meet design goals but also achieve better overall performance. The works on this approach include AMODA [6], ASTRX/OBLX [18], STAR [19], Anaconda [5], as well as commercial synthesis tools [27-28]. Our proposed method falls into this category.

## Cases study: charge-pump phase locked loop

In this paper, we utilize Charge Pump Phase-Locked Loop (CPPLL) as a concrete example. CPPLL is a representative mixed-signal circuit.  It is wildly used in a variety of fields, such as frequency synthesis, skew cancellation, clock generation and recovery, chip synchronization [26-29]. In this section, we briefly discuss the basic structure of CPPLL.

In Fig. 3, the block diagram displays the classic structure of a third order CPPLL. It consists of 5 fundamental blocks, namely Phase Frequency Detector (PFD) (Fig.2 (a)), Charge Pump (CP) (Fig.2 (b)), Loop Filter (LP) (Fig.2 (c)), Voltage Controlled Oscillator (VCO) (Fig.2 (d)) and Frequency Divider (FD). These five functional blocks form a negative feedback system, locking the phase and frequency of the feedback signal fb to a reference signal fi. This locking behavior is achieved by comparing fi and fb for many iterations. Once fb matches fi, CPPLL generates an output frequency fo which is N times of fi. Otherwise, CPPLL continues to compare two signals.

# III.		Overview of the proposed methodology

Fig.4 displays the flowchart of the proposed hierarchical optimization based technology migration methodology. The inputs include three essential elements: Process Design Kit (PDK) of target technology, source design and design constraints.  Source design provides original circuit netlist, specs and their test benches. The circuit netlist is a fine tuned

Fig.4 Flowchart of the proposed method

circuit schematic in source technology. The design specs their testbench describe what data should be collected and how it should be analyzed, as well as what input should be applied to stimulate   the    circuit.   Constraints   include   design    specs, functional conditions (e.g. transistor is working in the saturation region), as well as design variables and their variational ranges. The proposed migration method generates the target design which complies with all the desired performance accounting for both process variation and post layout parasitics.

The flowchart is briefly described as follows and then discussed in details later.

Step 1: Primary schematic migration. The MGO based resizing procedure focuses on one block represented in TL, while other blocks are replaced with BL models. It primarily resizes the original design block by block in target technology.

Step 2: Schematic migration refinement. In this step, all the blocks are represented in TL. Starting from the primary design determined from Step 1, another round of optimization based sizing procedure is conducted to further improve the solution.

Step 3: PVT variation evaluation. In this step, the generated solution is evaluated accounting for PVT variations. If it fails, it goes to Step 1.

Step 4: Layout parasitic evaluation. This step completes the layout and explores the parasitic effect. If it fails, it goes to Step 1.

## Hierarchical optimization based schematic migration

- Primary schematic migration

Simulating a TL mixed-signal system displays multiple challenges and is time demanding. A complex mixed-signal design can take days or even weeks to simulate.  Moreover, the simulation-optimization loop based method requires evaluating circuit characteristics for many iterations. Therefore, it is important to shorten the simulation time.

The primary schematic migration is performed block by block. At a time, the i-th analog block is represented in TL using BSIM4 model and set as the optimization object; the preceding i-1 resized blocks are presented with updated BL models with parameters calibrated by the retargeted TL designs; the following n-i blocks are presented using the original behavioral level models with parameters extracted from source design. By using behavioral models, the resultant system can be efficiently simulated in a shorter time without compromising the systematical functionality.

The n blocks can be either analog or digital blocks. For digital blocks, the migration is either automatically synthesized from RTL description or manually created using standard cells. As a result, the migration effort mainly focuses on complex and technology dependent analog blocks.

- Behavioral model creation and replacement

Behavioral models are created to capture the circuit functionality and input-output relationship. The block characteristics are parameterized and parameters are extracted from TL simulations. The created behavioral model is more flexible than TL model. It is easier to maintain, reuse, configure and migrate between different technologies.

The behavioral model is implemented using Verilog-AMS language [31] in this context. Verilog-AMS language, as an extension of traditional Verilog HDL language, is intended to support BL modelling for mixed-signal system. By using Verilog-AMS, BL blocks are written into event driven models in terms of ports and external parameters. The modules are connected with TL blocks to ensure a continuous and smooth simulation of the whole mixed-signal system.

The behavioral model of PFD using Verilog-AMS is presented in Fig.5. It is based on a well-known ideal state machine diagram depicted in [32]. PFD measures fi with fb to produce two signals: up and dn. up and dn are preset to be high and low, respectively. When fi leads fb, up generates a pulse, while fi lags fb, dn produces a pulse. The pulse width is proportional to phase difference between fi and fb, and it becomes larger as the phase difference increases.

The parameter state in Fig.5 is utilized to record the difference between fi  and fb. When the rising edge of fi or fb occurs, an event is triggered and conditionally changes the state value. There are four model parameters are extracted to fulfill the behavioral model.  td_up and td_dn capture the delay time of input to up and dn outputs, while trise and  tfall variables denote the rising and falling time of signal transitions. These variable values are obtained initially through the transient analysis of original design. After the primary resizing procedure, model parameters are adaptively modified.

module PFD (fi,  fb, up, down)

……

parameter real  ttol;            // accuracy tolerance,

parameter real  trise, tfall ;   // rising, falling signal transition time

real td_up, td_dn;                  // delay time from input to up, down

……

analog begin

@(initial_state) begin

state=0;

reset=0;

@(cross( V(fi) - Vdd/2 , 1 , ttol )) begin

if ( state = -1 ) reset=1 else state=1;   // Event block

end

@(cross( V(fb) - Vdd /2 , 1 , ttol)) begin

if ( state =1 ) reset=1 else state = -1;  // Event block

end

@(timer(reset_delay)) begin

if (reset==1) state=0, reset=0;

end

V(down) <+ transition( ((state ==1) || (reset==1))? Vdd : 0, td_dn , trise, tfall);

V(up) <+ transition( ((state ==-1) || (reset==1))? 0: Vdd , td_up , trise, tfall);

endmodule

Fig.5 Behavioral description for PFD using Verilog-AMS

module  VCO  (Vctrl,  out)

……

parameter real

Vamp, Vos, trise, tfall ;   // amplitude, offset of Vout

k1, k2, k3, k4 ;           //  curve fitting coefficients of VCO gain

…..

analog begin

frequency(out)= f(k1, k2, k3, k4, V(Vctrl));

@(cross(phase + ‘M_PI/2, +1, ttol) or cross(phase - ‘M_PI/2, +1, ttol)) begin

n=(phase >= -‘M_PI/2)&&(phase < ‘M_PI/2);

end

V(out) <+ transition(n? Vamp : 0, 0, trise , tfall ) + Vos ;

Phasenoise=noise_table(frequency(out));

end

endmodule

Fig.6 Behavioral description for VCO using Verilog-AMS

In order to validate the BL model of PFD, we compare the performance of two PFDs with and without BL models. In Fig.7, when fi leads fb, the BL up signal (pink dotted line) well captures the transition behaviors of its TL counterpart (blue dotted line).  In Fig.8, We further validate the PFD BL model through the settling behaviors of the whole CPPLL. The one with BL PFD model (black dashed line) exhibits great consistency against its TL counterpart (blue solid line).

The behavioral model for VCO in Verilog-AMS is shown in Fig.6. VCO is to generate square wave with a specific frequency responding to different Vctrl values. The gain of VCO is a critical concern. The fb- Vctrl transfer curve is accurately modeled using curve fitting with the third degree polynomial. k1, k2, k3 and k4 are fitting coefficients, which are originally extracted from source design and then adjusted accordingly after VCO is sized. Fig.9 compares VCO gains of BL (blue line with ‘o’ mark) and TL (black dotted line) models. These two curves show great consistence.  Variables Vamp and Vos denote the amplitude and offset voltage of VCO output, respectively. The noise behavior is modeled with the built-in ‘noise_table’ function, which contains a series of frequency-power pairs. The frequency-power pairs are obtained though noise analysis originally from source design and then updated after VCO is primarily sized through stage 1. In Fig.10, we validate the BL noise model, which consists with TL phase noise simulation.

Fig.7 Timing diagram for BL and TL PFD comparison

Fig.8 BL and TL locking time comparison

Fig.9 BL and TL Vctrl and fb transfer curve of VCO comparison

Fig.10 BL and TL phase noise of VCO comparison

Table.1 Simulation Runtime Comparison for CPPLL with and without Behavioral Models

|  |  | Simulation Time (s) | Speedup |
| --- | --- | --- | --- |
| TL CPPLL | 179.92 |  |
| CPPLL with PFD BL model | 93.83 | 1.91× |
| CPPLL with CP BL model | 57.87 | 3.1× |
| CPPLL with VCO BL model | 3.75 | 48× |

By using behavioral modeling techniques, a complex mixed- signal system can not only be efficiently simulated with good accuracy, but also within a reasonable runtime cost. In Table.1, the runtime costs of the CPPLLs with different behavioral  models are compared with their TL CPPLL counterpart. The data are the mean values from 100 simulation runs of each CPPLL. The CPPLL employing PFD, CP and VCO behavioral models achieve averagely 1.91×, 3.1× and 48× speedup over the TL CPPLL, respectively. The speedup in simulation time makes feasible and promising to utilize BL model in the optimization iterations.

- Constrained NLP problem formulation

The primary schematic migration of i-th block is finished by applying MGO. It aims at generating a design point which satisfies all the design requirements. For solving the problem, a Non-Linear Programming (NLP) problem is formulated:

maximize/minimize           F=fp

subject to           p-≤p≤p+

Cjp≤0,                        (1)

where p indicates an n dimensional parameter space, which maps into a response space F; f∙ formulates the objective function in terms of p. Cj∙≤0, j=1,2,…,M defines a set of nonlinear inequalities, describing design restrictions.

p is constituted of n device parameters, such as geometric sizes of transistor, resistor and capacitor, biasing voltage and current. For a certain circuit design under a specified technology node, each pi is usually bounded by [pi-, pi+]. pi- and pi+denote lower and upper bounds, respectively. For example, typical maximum and minimum transistor dimension is predefined by an IC foundry or user-defined; biasing current is limited by power consumption requirement.

A feasible solution to the NLP problem is a parameter set that makes all the constraints hold (meets all the design specs). An optimal solution is the feasible solution with largest F value. F can be either a single or combined performance responses. In this paper, we use an auxiliary equation which combines design specs together. The absolute difference between the measured characteristic value and its corresponding design spec is normalized with respect to design spec and summed together:

Feval=m=1Msign×abs(ymeasurem-yspecmyspecm)                     (2)

sign=1,        if ymeasurem is better than yspecm-1,                                           otherwise

where M indicates the number of design specs; ymeasurem and yspecm are the m-th measured characteristic and spec. When the measured value is better than spec, it is added to Feval. Otherwise, it is subtracted from Feval. The lager the Feval is, the better the overall performance is.

The aim of solving (1) is to find the optimal solution. However, it is not trivial. There exist several challenges. First, for most real engineering problems, an explicit expression which precisely relates a circuit characteristic to device parameters is not simple and straightforward. It is almost never available and generally implicit. Consider the phase noise of CPPLL, although a few simplified mathematical models are proposed for expressing the phase noise in terms of circuit parameters [33], they are inferior to SPICE-like simulations in terms of accuracy. Thus, we run SPICE simulation to evaluate the circuit characteristics. Second, it is due to the high dimensionality of parameter space and the complicated response surface.

For better illustration, we firstly vary two parameters: biasing current Ibias of charge pump (Fig.2(b)) and channel width WMc2 of VCO (Fig.2(d)) in source technology. Assume that Ibias varies between [1, 5] μA, and WMc2 is within [1, 5] μm. In this example, WMc1 of VCO is set to be twice of WMc2, attempting to achieve comparatively symmetrical rising and falling transitions. In Fig.11 (a), a two dimensional parameter space constituted by Ibias and WMc2 is shown. We randomly generate a few of sampling points in the parameter space marked as blue dots. Fig.11 (b) visualizes how the CPPLL features vary with Ibias and WMc2. Each point in parameter space corresponds to a different CPPLL design, yielding different F response. Fig.12 shows the contour graph of the Fig.11 (b).

From Fig.11 and Fig.12, we can see that there are many feasible solutions available which are marked with dark red. Among all the feasible solutions, there exists a globally optimal solution. The global optimum is mostly desired solution.  We randomly sample a start point 1 in the parameter space shown in Fig.12. Then, a local search routine is triggered from it.  The local search discovers a local optimum rather than the global optimum. A “good” start point can help the numerical optimization routine to find the global optimal solution faster. Otherwise, it is easily confined in a local solution.

The simplest way to break the limitation is to restart the search routine at different regions. Therefore, MGO is a suitable strategy.  MGO is a very famous global optimization strategy. Its procedure is briefly shown in Fig.10. Generally, it includes two phases: global and local phases. In global phase, a number of start points are generated which are uniformly distributed in the parameter space. In local phase, a local search routine starts from one of the start points, and then follows a path to arrive at a local optimum. The start points are produced to discover as many local optima as possible. Then, the best one among them is considered as the global optimum. In theory, as the number of visited point approaches infinity, the probability of finding a global optimum solution is close to one [34].

The easiest method to generate start points is to generate in a pure random manner. However, pure random sequences are prone to form cluster, which leads to the same local optimum is obtained repeatedly. This phenomenon is shown in Fig.12 by using start point 1 and 3. The diversity of start points plays an indispensable role in seeking a globally optimal solution. We generate a quasi-random sobol sequence [35] to be the initial points. Sobol sequence belongs to Quasi Monte Carlo (QMC) method family. Sobol sequence is more uniformly distributed over the parameter space than the sequence generated through a purely random way. What is more, when more start points are needed, the successively generated start points “know” about the position of their predecessors and fill the gaps left previously. This prevents the same region from being repeatedly searched and the same local optimum from being discovered many times. Thereby, it increases the chances of finding the global solution.

After initial points are generated, local searches are invoked   from each of them. In general, the constrained NLP problem (1) is firstly converted into an unconstrained NLP problem using

Initial i =1

while stopping condition is not satisﬁed do

/*Global Phase*/

Generate m start points over parameter space p using QMC method

/*Local Phase*/

for each start point  pi  do

Apply a local search (L-BFGS) to improve  pi

i=i+1

end for

end while

Fig.10 Brief procedure of MGO

Fig.11 (a) Parameter space and (b) Performance space in terms of Ibias and WMc2

Fig.12 Contour of performance space

Lagrange function [36]:

maximum  Lp,λ=Fp+j=0mλjCjp,                (3)

where λj is Lagrangian multiplier. Therefore, equation (3) rather than equation (2) is solved iteratively. The successive solutions will eventually converge to the solution of the original constrained NLP problem. During optimization, both p and λ are combined into one variable vector X, which will be refined iteratively to obtain the optimal value.

For local search, one can choose evolutionary based optimization algorithms, like generic, simulated annealing, or gradient based optimization algorithms, for example, steepest descend, conjugate and quasi-newton algorithms. In our methodology, we use the classic Limited-memory Broyden Fletcher Goldfarb Shanno (L-BFGS) algorithm [37]. In each iteration, both the values of objective function and partial derivatives with respect to all the variables are updated. These values are accumulated with values from finite preceding iterations to approximate the inverse Hessian matrix. More specially, it defines sk=Xk-Xk-1 and yk=∇Lk-∇Lk-1, where Xk and ∇Lk are the ordered set of parameters and the gradient of the objective function L at the k-th iteration, respectively. Then the approximated inverse Hessian matrix Hk is updated by

Hk=I-skykTykTskHk-1I-ykskTykTsk+skskTykTsk               (4)

and the new point for the next iteration is given by Xk+1=Xk-Hk∇Lk.

The first-order partial derivative of L with respect to each Xkis calculated numerically through backward difference approximation:

∇LXk=LXk-LXk-∆Xk∆Xk .                           (5)

The information of gradient is important that it describes the variation trends in the nearby area of Xk.

Two main features of MGO can be concluded. First, it provides diversification which overcomes local optimality. The evenly distributed start points trigger design explorations at different regions. Compared with searching solely from the scaled source design, MGO adds more possibilities of obtaining better designs. Second, it shows quick convergence.  Gradient based optimization algorithm is applied, which guides the searches to follow the steepest increasing/decreasing direction towards the optimum.

## Schematic migration refinement

Note that Step 1 trades the accuracy with efficiency by using behavioral models. In Step 2, the accuracy is compensated by taking another optimization round to further improve the result obtained from Step 1.

As a result, another NLP problem is formulated. Table.2 compares two NLP problems in Step 1 and 2. The major difference is that Step 1 performs a block-level optimization in system-level connection, while Step 2 is a system-level optimization. In Step 1, the optimization objective and variables are from the block under optimization. Both block and system-level specs are used as constraints for both steps. For example, if the VCO block is under migration, the NLP problem in Step 1 can be configured as:

- Optimization objective: phase noise of VCO;

- Variables:  device dimensions of VCO;

- Constraints: block-level specs, such as VCO gain, transistor

Table.2 Comparison of NLP Problems in Optimization Stage 1 and 2

|  | Primary Schematic Migration | Schematic Migration Refinement |
| --- | --- | --- |
| Property | - Block-level optimization in a system-level connection - BL-TL co-simulation | - System-level optimization - TL simulation |
| Objective | Block-level spec | System-level spec |
| Variables | Block-level | System-level |
| Constraints | Both block and system level | Both block and system level |
| Algorithm | MGO | Local optimization algorithm |
| Stopping Criteria | - Meet all the specs and constraints - Norm change of two consecutive parameter sets - Objective function change - Maximum number of iterations |  |

operation range; system-level specs, e.g. the overall phase noise, power consumption and locking time.

On the other hand, in Step 2, system-level spec is optimized

subject to the constraints defined by both local and global design requirements. The variables are chosen over the whole system rather than a single block. Similarly, the NLP can be created in Step 2 as:

- Optimization objective: phase noise of CPPLL;

- Variables:  device dimensions of VCO, CP and LP;

- Constraints: block-level specs, e.g. bandwidth of LP, VCO gain, transistor operation range; and system-level specs, like the overall phase noise, power consumption and locking time.

For the optimization algorithm, Step 1 employs MGO and Step 2 can apply any suitable optimization algorithm. We choose a local optimization algorithm, like L-BFGS algorithm, to search from the initial point obtained from Step 1. The stopping criteria are the same for both Step 1 and 2 NLP problems.

The initial point plays critical role in the action of optimization. It greatly assists in quick convergence and locating the global optimum. When MGO is firstly conducted to every block in Step 1, it generates a good initial point for the system-level optimization of Step 2.

## PVT aware evaluation

After the hierarchical optimization based schematic migration, an optimal parameter set is obtained. However, there are several aspects that have to consider. Its robustness with respect to PVT variations has to be verified. The circuit characteristics are usually monotonic to VT variations. Their worst-case performance due to VT variations can be evaluated at extreme conditions of VT. For process variations, Monte Carlo (MC) method is always the golden standard, although there are a lot of non-MC methods are available in the literature.

In this evaluation step, MC method using Latin hypercube sampling is applied to evaluate the circuit performance robustness accounting for process variations. Latin hypercube sampling requires one fourth of the number of simulations needed by traditional MC method [44].   The required number of traditional MC samplings can be obtained [45]:

NMC=(CσY-Y)2∙Y(1-Y)                                (6)

where Y is the actual yield value, Y is the estimated yield value, Cσis the confidence level. For a 99.7% confidence level (Cσ=3), an error Y-Y=±1% and a yield of 90%, NMC is 8100.  To achieve the same accuracy, the number of Latin hypercube samplings is 2025.

After PVT evaluation, if the circuit character fails to meet the specs. Then performance deviation information is fed back to Step 1 to modify the design constraints. Then a new migration iteration starts.

To avoid too many iterations between Step1 and 3, Step 2 is modified into a process variation aware optimization process. The condition that all the specs are met at process corners is used to further constrain the optimization procedure in Step 2.    Note that the process corners are corresponding to ±3σ values of the joint probability density function of process parameters. It is prone to give a pessimistic analysis which leads to over-design. A more accurate but time-consuming method is to include MC method in the optimization process.

## Parasitic aware evaluation

Besides process variations, layout parasitics also have significant impact on circuit performance. Parasitics arise from transistor source/drain capacitance, resistance and capacitance in the interconnect wires [33].  Therefore, in our proposed migration flow, we also evaluate the parasitic effect.

In this step, the layout is developed and parasitic effect is explored using commercial Layout Parasitic Extraction (LPE) tool [46]. Then post layout evaluations are performed to further verify whether the circuit performance meet the specs or not. If the parasitic evaluation fails, the extracted parasitic resistors, capacitors are reused and constrain the next iteration in Step 2. In the next iteration, the computed parasitics are added to the netlist and travels the whole optimization procedure.

# IV.		Experimental Results

## Experiment settings

The proposed methodology for design migration is validated by the CPPLL shown in Fig.2. The CPPLL is used in a NFC system [30], which is originally implemented in UMC 130 nm CMOS technology with 1.2 V Vdd. This CPPLL is to provide 32 MHz output frequency.  It is firstly migrated to UMC 65 nm and then to IBM 65 nm CMOS technology with 1.1V Vdd.

The circuit characteristics of source design are listed in the second column of Table.4. They are set as the specs for target design. The migration for PFD and FD is based on digital library cells in target technology. The main migration task focuses on CP, LP and VCO blocks.

MGO algorithm is implemented using C\C++ language based on SOBOL library [38] for generating start points and ALGLIB library for L-BFGS [39]. All the circuits are simulated using HSPICERF. All the experimental data has been run on a PC with 12GB RAM and Linux operating system.

## Efficiency of MGO

In this experiment, CP, LP and VCO are represented in TL. MGO algorithm is performed to retarget CPPLL from UMC 130 nm to UMC 65 nm CMOS technology. Phase noise of CPPLL is set as the optimization objective. It is computed through the built in harmonic balance (HB) and phase noise (PNOISE) analysis of HSPICERF. The considered design parameters include every transistor dimension (length, width,

Fig. 13 Evaluated objective function values with number of iterations

Table.3 Comparison of Simulation Iterations

| Optimization Method | # of iteration | Speedup | Memory Usage (G Bytes) |
| --- | --- | --- | --- |
| MC | 2036 | Ground Truth | 0.2027 |
| PS | 1617 | 1.26× | 0.3536 |
| DE | 1048 | 1.94× | 0.1693 |
| GA | 1172 | 1.73× | 0.3542 |
| MGO | 124 | 16.42× | 0.1381 |

number of finger and multiplier) of CP and VCO, biasing current, and dimension (width, length and multiplier) of C1, C2 and R of LP. Variables are bounded by the minimum and maximum values allowed in target technology. Both of C1 and C2 are bounded that are less than 15p for area saving purpose.

We have compared MGO algorithm with Genetic Algorithm (GA) [40], Pattern Search (PS) [41], Difference Evolution (DE) [42] and MC in terms of computation cost. GA, PS and DE are all famous optimization methods. They do not require gradient information of the objective function. The MATLAB built-in GA and PS algorithms are used, while the implementation of DE in [43] is utilized. GA randomly chooses a set of initial points from the parameter space and updates them through many iterations. In sake of fairness, we start all the algorithms from the same start point.

The compared results are shown in Fig.13 in terms of evaluation value Fevalwith respect to simulation runs. Each visited parameter vector is used to calculate the evaluation value Feval. The result obtained from MC space is used as the ground truth, which uniformly samples 2000 points in parameter space. The pink dash line shows the result of MC that does not converge. Three stopping criteria are utilized for other approaches: the design specs and constraints are satisfied, the maximum number of function evaluation runs (2000 runs in this case) is met and the norm change of two consecutive searched parameter sets or change of objective function is less than the pre-defined tolerance (e.g. 10-6). GA evolves 40 generations with population size of 50. Other settings are using default values. GA converges to a local optimum as shown in red line with ‘o’ mark.  MGO (Green dotted line), DE (blue line with ‘*’ mark) and PS (black line) converge to local optima. From both Fig.13 and Table.3, we can see that MGO exhibits superior performance than other optimization methods. It quickly locates the global optimal through only 124 function evaluations from two start points.

Table.4 Performance for CPPLL under Different CMOS Technologies

| Design Specification | UMC 130nm | UMC 65nm | IBM 65nm |
| --- | --- | --- | --- |
| Phase Noise (dBc/Hz @ 600kHz) | -115.2 | -118 | -117.2 |
| Locking time (µs) | 4.7 | 2.6 | 4.2 |
| Min/Max VCO frequency (MHz) | 5-77 | 5-83 | 5-83 |
| Power consumption (mW@32MHz) | 0.142 | 0.105 | 0.098 |
| Phase margin (Degree) | 45 | 46 | 45.5 |
| Static Phase Offset (ps) | 10 | 9.3 | 9.8 |
| RMS Jitter(ps) | 20 | 14.3 | 18.8 |
| Area (um2) | 6525 | 4400 | 4125 |

1.4 ×

Time (h)

1.63 ×

1.38 ×

UMC65

IBM65

Fig. 14 Simulation time comparison with and without Step 1

That is averagely 16.42× faster than MC. When more start points are evaluated, better performance is obtained than all other optimization algorithms. DE is a derivative method of GA, and it shows very similar performance as GA. PS has inferior performance compared with other considered optimization methods. Note that due to the heuristic property of all the methods, the recorded numbers are the mean value from 10 runs of each algorithm. The memory usage of all considered optimization algorithms are compared and tabulated in the last column of Table.3.

## Validation of the proposed migration methodology

The resulting design has comparable or even superior performance to source design. Table.4 tabulates the overall performance of CPPLL in both source and two target technologies.

We have compared the runtime with and without Step 1 in Fig.14. Note that the recorded runtime only includes Step 1 and Step 2. Since we assume we use the same amount of time for PVT and parasitic evaluations. For porting to UMC65, only one round of migration process is conducted. Step 1 takes 1.2 hours, Step 2 takes 3.2 hours and it totally takes 4.4 hours. However, if MGO is directly applied to migrate TL CPPLL rather than performing Step 1 first, 6.2 hours are required to achieve the same result.

For porting to IBM65, it takes almost the same time (1.2 hours in Step1 and 3.1 hours in Step 2) as porting to UMC65. However, after the parasitic evaluation of Step 4, there is 17% performance degradation noticed at the fastest frequency that VCO can achieve. Then we proceed to iteration 2. In Step1, we tighten the highest VCO frequency spec by 17% faster. In Step 2, the parasitic information is added to the original netlist and

σ=4.7

σ=9.72

###### (b)

Fig. 15 Histograms of phase noise of UMC 65 (a) without and (b) with considering process corners

evaluated during the whole optimization process. Since the parasitic aware netlist contains much more components, it requires more SPICE time to solve. As a result, totally 13.2 and 18.8 hours are needed with and without Step 1, respectively. Step 1 reduces the complexity of design exploration for Step 2, it generates a good initial point for Step 2.

For robustness analysis accounting for PVT variation, we randomly run 2025 samplings using Latin hypercube technique in the parameter space. The statistical distribution for phase noise is shown Fig. 15 (a) and (b). In Fig.15 (a), Step 2 does not consider the process corner information, which results in a 9.72 dBc/Hz deviation to phase noise at 600 kHz offset frequency. In the other hand, by taking into consideration of process corners in the Step 2 optimization flow, the deviation is significantly reduced.

# IV.		Conclusion

A hierarchical optimization-simulation loop based methodology is proposed for process migration automation of mixed-signal system. The methodology can retarget a mixed-signal design in reasonable iterations while giving the same or better performance. The migration time cost is dramatically reduced by employing the behavioral models and MGO in the first round of optimization. A second round of local optimization is followed for better performance. To obtain a robust result, the process corner is taken into account in the optimization procedure, and the parasitic effect is also considered for obtaining a parasitic closure design. The proposed methodology has been proved useful and efficient for technology migration and design reuse through a concrete CPPLL system example.  This general methodology can be extended to other mixed-signal systems.

By taking good use of modern multi-processors computer structures   or   distributed   workstation network, the inherent parallel computing property of MGO can be greatly explored to further speed up the proposed technology migration of mixed-signal circuit approach.

References

- H. Bakeer, O. Shaheen, H. Eissa and M. Dessouky, "Analog, Digital and Mixed-Signal Design Flows", in International Design and Test Workshop, 2007, pp.247-252.

- M. Naumowicz, M. Melosik and P. Katarzynski, "Technology Migration of Analogue CMOS Circuits Using Hooke-Jeeves Algorithm and Genetic Algorithms in Multi-Core CPU Systems," in Proc. Int. Conf. on MIXDES, 2013, pp. 267-272.

- R. Phelps, M.J. Krasnicki, R.A. Rutenbar, L.R. Carley, and J.R. Hellums, "A case study of synthesis for industrial-scale analog IP: redesign of the equalizer/filter frontend for an ADSL CODEC," in IEEE Design Automation Conf., 2000, pp. 1 – 6.

- S. Funaba, A. Kitagawa, T. Tsukada and G. Yokomizo, "A Fast and Accurate Method of Redesigning Analog Subcircuits for Technology Scaling," J.  of Analog Integrated Circuits and Signal Processing, vol.25, pp. 299-307, 2000.

- R. Phelps, M. Krasnicki, R.A. Rutenbar, L.R. Carley, J.R. Hellums, "Anaconda: simulation-based synthesis of analog circuits via stochastic pattern search, " IEEE Trans. On Computer-Aided Design Inst. Circuits Syst., vol. 19, no. 6 , pp. 703-717, 2000.

- P. Wu, R. Mack, R. Massara, D. Bensouiah  and A. Kemp, “AMODA: a flexible framework for automatic migration of analog macro designs using optimization techniques”, in Proc. of IEEE Midwest Symposium on Circuits and Systems, pp. 988–991, 2000.

- A. Savio, L. Colalongo, M. Quarantelli, and Z.M. Kovacs-Vajna, "Automatic Scaling Procedures for Analog Design Reuse," IEEE Trans. on Circuits and Systems I, vol. 53, pp. 2539 - 2547., 2006.

- K. Francken and G. Gielen, "Methodology for analog technology porting including performance tuning," in Circuits and Systems, IEEE International Symposium on, vol. 1, Jul 1999, pp. 415–418.

- S. Hammouda, H. Said, M. Dessouky, M. Tawfik, Q. Nguyen,W. Badawy, H. Abbas, and H. Shahein, "Chameleon ART: a nonoptimizationbased analog design migration framework," in Design Automation Conference, 43rd ACM/IEEE, Jul 2006, pp. 885–888.

- R. Acosta, F. Silveira, P. Aguirre, "Experiences on Analog Circuit Technology Migration and Reuse, " in Proc. XV Symposium on Integrated Circuits and Systems Design, 2002, pp. 169–174.

- T. Yang, M. Gao, S. Wu and D. Guo, "A new reuse method of analog circuit design for CMOS technology migration, " in Proc. Int. Conf. on ASID, 2010, pp. 112-115.

- A. I. A. Cunha, M. C. Schneider, C. Galup-Montoro, "An MOS Transistor Model for Analog Circuit Design, " IEEE J. of Solid-State Circuits, vol. 33, no. 10 , pp. 1510–1519, 1998.

- C. Galup-Montoro, M.C. Schneider, R. Coitinho, "Resizing Rules for MOS Analog-Design Reuse", IEEE Design and Test of Computers, pp. 50-58, April 2002.

- T. Levi, J. Tomas, N. Lewis, P. Fouillat, "A CMOS resizing methodology for analog circuits: linear and non-linear applications”, IEEE Journal Design and Test of Computer, vol. 26, pp. 78-87, January-February 2009.

- A. Graupner, J. Roland, and W. Reimund, "Generator Based Approach for Analog Circuit and Layout Design and Optimization, " in Design, Automation & Test in Europe Conference & Exhibition(DATE), 2011, pp. 1–6.

- M. Dessouky, A. Kaiser, M.-M. Louerat, and A. Greiner, “Analog design for reuse-case study: very low-voltage Delta-Sigma modulator,” in Design, Automation and Test in Europe, Conference and Exhibition, 2001, pp. 353–360.

- T. Levi, N. Lewis, J. Tomas and S. Renaud, "Application of IP-based Analog Platforms in the design of Neuromimetic Integrated Circuits", IEEE Trans. On Computer-Aided Design Inst. Circuits Syst., vol. 31, Issue 11, pp. 1629-1641, 2012

- R. Rutenbar, G. Gielen, B. Antao, "Synthesis of High Performance Analog Circuits in ASTRX/OBLX," IEEE Trans. On Computer-Aided Design Inst. Circuits Syst., vol. 15, no. 3, pp. 273-294, Mar 1996.

- D. Liu, S. Sidiropoulos, and M. Horowitz, "A framework for designing reusable analog circuit, " in Int. Conf. on Computer-Aided Design, pp. 375-380, Nov. 2003.

- J.B. Grimbleby, "Automatic analogue circuit synthesis using genetic algorithms," Inst. Elect. Eng. Proc.  Circuits Devices Systems, vol. 147, 2000,  pp. 319-323.

- M. Davis, L.Liu, and J.G.Elias, "VLSI circuit synthesis using a parallel genetic algorithm," in Proc. IEEE Conf. on Evol. Comput., 1994, pp. 104-109.

- T.Sripramong and C. Tomazou, "The invention of CMOS amplifiers using genetic programming and current-flow analysis,” IEEE Trans. On Computer-Aided Design Inst. Circuits Syst., vol. 21, no. 11, 2002, pp. 1237-1252.

- J.R.Koza et al., "Automated synthesis of analog electrical circuits by means of genetic programming,” IEEE Trans. Evol. Comput, vol. 1, no. 2, pp. 109-128, Jul. 1997.

- Cadence Virtuoso NeoCircuit- circuit sizing and optimization (2007, February 1). Available: http: // www. cadence. Com /products /custom_ic /neocircuit

- Synopsys CustomExploer- complete transistor-level analysis and debugging environment (2011, April 1). Available: http: //www .synopsys .com/ products/ mixedsignal/hspice/circuit_explorer

- F. Gardner, “Charge pump phase-lock loops,” IEEE Trans. Commun.,vol. COM-28, Nov 1980, pp. 1849–1858.

- I. Maneatis, "Self-Biased, High-Bandwidth, Low-Jitter 1-4096 Multiplier Clock-Generator PLL," IEEE J. of Solid-State Circuits, vol. 38, no. 11, 2003, pp. 1795–1803.

- A. Loke et al., "A Versatile 90-nm CMOS Charge-Pump PLL for SerDes Transmitter Clocking," IEEE J. Solid-State Circuits, vol. 41, no. 8, pp. 1894-1907, Aug. 2006.

- D.M. Fischette, A.L.S. Loke, M.M. Oshima, et al., “A 45nm SOI-CMOS Dual PLL Processor Clock System for Multi-Protocol I/O,” ISSCC Dig. Tech. papers, pp. 246-247, Feb., 2010.

- Standard ECMA-340, Near Field Communication Interface and Protocol (NFCIP-1)http://www.ecmainternational.org/publications/files/ECMA-ST/Ecma-340.pdf - 2nd Edition / December 2004.

- Verilog-AMS Language Reference manual: analog & Mixe-signal Extension for verilog-HDL(2009, June 1). Available from http:// www.accellera.org/downloads/standards/v-ams/VAMS-LRM-2-3-1.pdf

- B. Razavi, Design of Monolithic Phase Locked Loops and Clock recovery Circuits – a Tutorial, IEEE Press, April 18, 1996.

- F. Herzel, S. Osmany and J. Christoph, "Analytical phase noise modeling and charge pump optimization for fractional-N PLLs," IEEE Trans. on Circuits and Systems I, vol. 57, pp. 1914 – 1924, 2010.

- U. Zsolt, L. Lasdon, J. Plummer, F. Glover, J. Kelly, and R. Martí, “ Scatter Search and Local NLP Solvers: A Multistart Framework for Global Optimization,” INFORMS Journal on Computing, Vol. 19, No. 3, 2007, pp. 328–340

- Sobol,I.M, "Distribution of points in a cube and approximate evaluation of integrals," in U.S.S.R Comput. Maths. Math. Phys. vol. 7, 1967, pp. 86–112.

- D. Bertsekas, “Constrained Optimization and Lagrange Multiplier Methods,” Athena Scientific, Belmont, MA, 1996.

- R. Byrd, P. Lu, J. Nocedal and C. Zhu, "A Limited Memory Algorithm for Bound Constrained Optimization," SIAM J. on Scientific and Statistical Computing, vol. 16 (5), 1995, pp. 1190–1208.

- The Sobol Quasirandom Sequence(2009, December 12). Available: http://people.sc.fsu.edu/~jburkardt/cpp_src/sobol/sobol.html

- ALGLIB User Guide (2013, November 1). Available: http: //www. alglib. net

- Goldberg, David E., Genetic Algorithms in Search, Optimization & Machine Learning, Addison-Wesley, 1989.

- Audet, Charles and J. E. Dennis Jr. "Analysis of Generalized Pattern Searches." SIAM J. on Optimization, vol. 13(3), 2003, pp. 889–903.

- R. Storn and K. Price, “Differential evolution-a simple and efﬁcient adaptive scheme for global optimization over continuous spaces,” TR-95-012, 1995 [Onlline]. Available: http://http.icsi.berkeley.edu/ ~storn/litera.html

- J. Ramos, CMOS Operational and RF Power Amplifiers for Mobile Communications. PhD thesis, K. U. Leuven, Department of Electrical Engineering, Belgium, March 2005.

- J.F. Swidzinski and Kai Chang, "Nonlinear Statistical Modeling and Yield Estimation Technique for Use in Monte Carlo Simulations [Microwave Devices and ICs] ", IEEE Trans. On Microwave Theory and Techniques, vol. 48, Issue 21, pp. 2316–2324, 2000

- H. Graeb, Analog Design Centering and Sizing. Berlin, Germany: Springer, 2007.

- Cadence QRC Extraction Solution (2014, April 14). Available: http://www.cadence. com/products/cic/quantus_qrc_extraction

This work was partially supported by SRC grants 525870 and 637163 and NSF grant 1115556; National Natural Science Foundation of China (NSFC) research project 61125401, 61376040 and 6122840; National Basic Research Program of China under the grant 2011CB309701, National major Science and Technology Special Project 2014ZX02301001-005-002; Shanghai Science and Technology Committee project 13XD1401100.

Liuxi Qian, Zhaori Bi and Dian Zhou are with the Department of Electrical Engineering, University of Texas at Dallas, Richardson, TX 75080 (e-mail: lxq080020@utdallas.edu).  Dian Zhou is a "Thousand People Plan” Professor, Fudan University, China.

Xuan Zeng is with ASIC & System State Key Lab., Microelectronics Department , Fudan University, Shanghai, P.R. China .